State Board of Technical Education &Training (TS)

Department of Technical Education

| Course Title | Theory of Structures | Course Code | CE-301 |
| --- | --- | --- | --- |
| Semester | III Semester | Course Group | Core |
| Teaching Scheme in periods(L:T:P) | 60:15:0 | Credits | 3 |
| Methodology | Lecture+Tutorials | Total Contact periods | 75 |
| CIE | 60 Marks | SEE | 40 Marks |

Pre requisites	 Knowledge of Structural Mechanics, Basic Engineering Mathematics

Course Outcomes	 On completion of the course, the student shall be able to

| CO1 | Analyze stresses and stability conditions of simple dams and retaining walls and compute earth pressure and base stresses using standard formulae. |
| --- | --- |
| CO2 | Determine slopes and deflections of beams subjected to point loads and uniformly distributed loads using the double integration method for standard support conditions. |
| CO3 | Evaluate beam deflections using Macaulay’s method and Mohr’s theorems for cantilever and simply supported beams under symmetrical loading. |
| CO4 | Analyze statically indeterminate beams such as fixed and continuous beams by applying standard fixed-end moment expressions and the Moment Distribution Method, and sketch bending moment diagrams. |
| CO5 | Determine internal forces in statically determinate pin-jointed trusses using the method of joints and method of sections for simple loading conditions. |
| CO6 | Assess the load-carrying capacity of columns and struts by calculating slenderness ratio and safe load using Euler’s and Rankine’s formulae, considering different end conditions. |

Course Content and Blue Print of Marks for SEE

| Unit No | Unit Name | Periods | Questions to be set for SEE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | R |  |  | U | A |
| 1 | Dams and Retaining walls | 13 | Q4 | Q1 |  | Q9(a) | Q13(a) |
| 2 | Deflection of beams-I | 12 |  |  |  |  |  |
| 3 | Deflection of beams-II | 12 |  | Q2 |  | Q10(a) | Q14(a) |
| 4 | Statically indeterminate beams | 13 |  |  |  |  |  |
| 5 | Stresses in Frames | 15 |  | Q3 | Q5,Q6 | Q9(b), Q11(a), Q11(b) | Q13(b), Q15(a), Q15(b) |
| 6 | Columns and Struts | 10 |  |  | Q7,Q8 | Q10(b), Q12(a), Q12(b) | Q14(b), Q16(a), Q16(b) |
| Total |  | 75 | 8 |  |  | 8 | 8 |

| CO | PO1 – Basic & Discipline Specific Knowledge | PO2 – Problem Analysis | PO3 – Design/Development of Solutions | PO4 – Engineering Tools, Experimentation & Testing | PO5 – Engineering Practices for Society, Sustainability & Environment | PO6 – Project Management | PO7 – Lifelong Learning | Linked POs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | 3 | 3 | - | 2 | 1 | 2 | 1 | 1,2,4,5,6,7 |
| CO2 | 3 | 3 | - | 2 | 2 | - | 1 | 1,2,4,5,7 |
| CO3 | 3 | 3 | - | 2 | 2 | - | 1 | 1,2,4,5,7 |
| CO4 | 3 | 3 | 3 | 2 | 3 | - | 1 | 1,2,3,4,5,7 |
| CO5 | 3 | 3 | 3 | 2 | 2 | - | 1 | 1,2,3,4,5,7 |
| CO6 | 3 | 3 | 3 | 2 | 1 | 1 | 1 | 1,2,3,4,5,6,7 |

COURSE CONTENTS

UNIT 1: Dams and Retaining walls               Duration: 13 Periods(L:7.8 – T:5.2)

Introduction – rectangular dams – trapezoidal dams (one side batter only) - Calculation of maximum and minimum stresses at the base of a dam - trapezoidal dams having water face vertical and inclined on one side only– Conditions for the stability of a dam

Active and passive earth pressure - Angle of internal friction, Angle of surcharge. Calculation of active earth pressure by Rankine’s formula without surcharge (with surcharge excluded). Calculation of maximum and minimum stresses at the base of retaining wall having soil face vertical with levelled earth. (Walls with batter on earth face not included). General conditions of stability of retaining walls – Middle third rule – Distribution of pressure on foundation of retaining walls –Problems on one side batter only other than earth face.

UNIT 2: Deflection of beams-I                        Duration: 13 Periods(L:7.8 – T:5.2)

Introduction – Deflected profiles of beams with different support conditions – Strength and stiffness of beams – Relation between curvature, slope and deflection - Slope and deflection for simply supported beams under symmetrical loading – Slope and deflection in cantilever beams under point load and udl- Double integration method – Derivation of standard cases –Problems.

UNIT 3: Deflection of beams-II                      Duration: 13 Periods(L:7.8 – T:5.2)

a) Macaulay’s method for slope and deflection–Simply supported beams under concentrated and uniformly distributed loads –Problems.

b) Mohr’s theorems for slope and deflection – Cantilevers and simply supported beams with symmetrical loading – Problems.

UNIT 4: Statically indeterminate beams       Duration: 13 Periods(L:7.8 – T:5.2)

Fixed Beams: Introduction- Determination of fixed end moments by Moment Area method – standard cases – fixed beams subjected to symmetrical concentrated loads – Fixed beams subjected to U.D.L throughout –B.M.D. and S.F.D – problems (without sinking of props) – Maximum deflection formulae of fixed beams subjected to central point load and fixed beams subjected to U.D.L throughout (No derivation) – problems.

Introduction to Moment Distribution Method – Sign conventions – stiffness factor – carry over factor – distribution factor – Application to continuous beams with same moment of inertia through out the span carrying single point load (central or eccentric) or UDL throughout the individual span of two span and three span beams only (combination of UDL and point loads not included together)-sketching of B.M.D only

UNIT 5: Stresses in frames	Duration: 13 Periods(L:7.8 – T:5.2)

Frames – Definition – classification based on number of members and number of joints – Determination of forces in members of statically determinate pin jointed trusses – method of joints and method of sections– application to simple trusses-(simply supported and cantilever) trusses under loads acting at the joints.

UNIT 6: Columns and struts                                       Duration: 13 Periods(L:7.8 – T:5.2)

Calculation of Least Moment of Inertia, radius of gyration and slenderness ratio for Solid circular, Hollow circular, Rectangular, I section and Built-up sections – Compression Members-Types and Classification - Short and Long columns, failures - Different end conditions - Effective length - calculation of safe load on columns with axial load only by Eulers and Rankine’s formula - Limitation of Euler’s formula

RecommendedBooks

- Strength of Materials by R. Subramanian , Oxford university Press

- Analysis of Structures by Thandavamoorthy, Oxford university Press

- Theory of structures by S.Ramamrutham and R.Narayan, Dhanpath Rai publications

- Strength of materials by B.C.Punmia, Laxmi publications

- Theory of structures by B.C.Punmia and A.K.Jain, Laxmi publications

- Structural Analysis –A Unified Approach by D S PrakashRao, Universities press publications

- Strength of materialsby R.K .Rajput, S.Chand publications

- Strength of Materials by R.K. Bansal,Laxmi publications

- https://www.sciencedirect.com/book/9780080116549/basic-theory-of-structures

Journals

- https://www.sciencedirect.com/journal/engineering-structures

- https://www.sciencedirect.com/journal/thin-walled-structures

- https://www.sciencedirect.com/journal/computer-methods-in-applied-mechanics-and-engineering

Suggested E-learning references

- https://archive.nptel.ac.in/courses/105/105/105105166/

- https://www.youtube.com/watch?v=HmQjDvcnKzo

- https://nptel.ac.in/

Suggested Learning Outcomes

On completion of the subject the student shall be able to

UNIT–1: Dams and Retaining Walls

- Explain the types of dams and retaining walls and their structural requirements.

- Calculate maximum and minimum stresses at the base of rectangular and trapezoidal dams.

- Determine active earth pressure on retaining walls using Rankine’s theory for standard cases.

- Apply stability conditions and the middle-third rule to dams and retaining walls.

- Draw pressure distribution diagrams and solve numerical problems on simple retaining walls.

UNIT–2: Deflection of Beams – I

- Explain deflected shapes, slope, and deflection of beams under different support conditions.

- Relate curvature, slope, and deflection in beams subjected to loading.

- Determine slope and deflection of simply supported beams under symmetrical loading.

- Calculate slope and deflection of cantilever beams under point load and uniformly distributed load using the double integration method.

UNIT–3: Deflection of Beams – II

- Apply Macaulay’s method to determine slope and deflection of simply supported beams under point and distributed loads.

- Use Mohr’s theorems to evaluate slope and deflection of cantilever and simply supported beams.

- Solve numerical problems related to beam deflection using standard methods.

UNIT–4: Statically Indeterminate Beams

- Explain the behavior of fixed and continuous beams.

- Determine fixed-end moments for fixed beams under standard loading conditions.

- Draw shear force and bending moment diagrams for fixed beams without support settlement.

- Apply the Moment Distribution Method to analyze two-span and three-span continuous beams and sketch bending moment diagrams.

UNIT–5: Stresses in Frames

- Classify pin-jointed frames and trusses based on members and joints.

- Determine forces in truss members using the method of joints.

- Apply the method of sections to find forces in selected truss members.

- Analyze simple simply supported and cantilever trusses under joint loads.

UNIT–6: Columns and Struts

- Calculate least moment of inertia, radius of gyration, and slenderness ratio for different column sections.

- Differentiate between short and long columns and identify modes of failure.

- Determine effective length of columns for various end conditions.

- Calculate safe load on columns using Euler’s and Rankine’s formulae and state the limitations of Euler’s theory.

Suggested Student Activities

- Analyse part of your institution building framed structure elements by Moment Distribution Method

- Prepare a report on identifying and analyzing the trusses found in nearby workshop

- premises of your institution

- Prepare the list of dams constructed in India from 2000 to 2025.

- Submit a detailed report on dams or barrages failures happened recently in India.

- Examine the retaining walls constructed at the hill roads.

- Evaluate the retaining walls constructed in the underground construction of the buildings or ramps

State Board of Technical Education and Training,Telangana

Model Question Paper

DCER III Semester Mid Semester-I Examination

Course Code: CE-301	Duration:1 hour Course Name: THEORY OF STRUCTURES	Max.Marks:20 Marks

# ---------------------------------------------------------------------------------------------------------------- PART-A

Answer all questions, Each Question carries one mark	4x1 = 4 Marks

- Define active earth pressure.

- State the middle-third rule for retaining walls.

- What is the relation between curvature, slope, and deflection in a beam?

- Name the method used to calculate slope and deflection in cantilever beams under point load or UDL.

# PART-B

Answer TWO questions. Each question carries THREE marks	2x 3 = 6 Marks

  - Sketch the stress diagram at the base of the dam, when the compressive stress is 150Mpa at heel and 50Mpa at toe of the dam.

(OR)

  - Summarize the importance of middle third rule from safety point of view of dam

6(a) Explain the difference between strength and stiffness of a beam.

(OR)

6(b) Determine the slope at the free end of a cantilever beam subjected to a point load at the tip using the double integration method

# PART-C

Answer TWO questions. Each question carries FIVE marks

7(a). A trapezoidal masonry dam 5m high, 1m wide at its top and 3m wide at its bottom retains water on its vertical face. What are the maximum and minimum stresses at the base when the reservoir is empty? Take ωm= 22kN/m3 and ωw= 9.81kN/m3.

# (OR)

7(b)	A dam of rectangular section having 10 m high and retains water up to the top of the dam height. Find the minimum base width to avoid tension at the base, if the Weight of masonry 25 kN/cu.m, and specific weight of water 10kN/cu.m

8(a)   A cantilever beam of length L=4 m is subjected to a point load of P=10 kN at its free end. The beam has a flexural rigidity of EI=2×106 N/mm2.

(OR)

8(b)    Derive a formula to calculate maximum slope for a simply supported beam with concentrated load at the centre, using Double integration method.

State Board of Technical Education and Training,Telangana Model Question paper

DCE III semester Mid Semester-II Examination

Course Code:CE-301	Duration:1 hour

Course Name: THEORY OF STRUCTURES	Max.Marks:20

# --------------------------------------------------------------------------------------------------------------- PART-A

Answer ALL questions, Each Question carries ONEmark	4x1 = 4 Marks

- State the ratio of active earth pressure to Passive earth pressure for the angle of repose 30 degrees.

- What is Surcharge?

- Define Propped support.

- Define point of contraflexure.

# PART-B

Answer TWO questions. Each question carries THREE marks	2x 3 = 6 Marks

  - A trapezoidal retaining wall 10m high, 3m top width retains earth on vertical face and has a slope of 1:2.5 on the other side. Calculate the distance of itscentre of gravity from the toe of the wall.

# (OR)

  - A trapezoidal masonry retaining wall 2m wide at top and 5m wide at bottom is 7m high. The vertical face retains earth having an angle of repose of 300 at a surcharge of 200 with the horizontal. Calculate the earth pressure.

  - A propped cantilever beam of span 4m carries a Point load of 22 kN at the mid span. Find the prop reaction.

# (OR)

  - A fixed beam of span 5m carries a UDL of 10kN/m over entire span. Calculate net positive bending moment.

# PART-C

Answer TWO questions. Each question carries FIVE marks	2x 5 = 10 Marks

# 7(a)	A cantilever of span 6m carries a UDL of 10kN/m for a length of 4m from fixed end   and a point load of 12 kN at the free end. Determine maximum slope and deflection at the free end by Moment area method. Given E = 200 kN/mm2, Ixx = 32 x 106 mm4. (OR)

7(b)	An I-section is used as a simply supported beam of span 5m to carry two point loads of 20kN each at a distance of 1.5m from both the supports. Find the position and magnitude of maximum deflection. Take E= 2 x 105 N/mm2, I=73.33 x 106 mm4. Use Maculay’s method.

# 8(a)	Derive a formula to calculate maximum slope and maximum deflection for a cantilever  beam with a point load ‘W’ at the free end, using Double integration method.

# (OR)

8(b) 	Derive a formula to calculate maximum slope for a simply supported beam with a udl of ‘w’ kN/m throughout the span, using Double integration method.

State Board of Technical Education and Training,Telangana Model Question paper

DCER III semester Semester End Examination

Course Code: CE-301	Duration:2 hours

Course Name: THEORY OF STRUCTURES	Max. Marks: 40 Marks

# PART-A

Answer allquestions.Each question carries one mark	8x1 = 8 Marks

- Write any two stability conditions of a dam.

- What is the difference between a dam and a retaining wall?

- Define a truss and draw a simple truss.

- What is meant by statically indeterminate beam?

- Define „Distribution Factor‟ in Moment Distribution Method?

- Draw the deflected shapes of a two span continuous beam.

- State the difference between perfect frame and imperfect frame.

- Mention any two assumptions made in the analysis of frames.

# PART-B

Answer FOUR questions. Each question carries three marks.	4 x 3 = 12 Marks

9(a)  If the magnitude of horizontal water pressure on the vertical back of a dam and the weight of the dam are 320kN and 920kN respectively. Determine the resultant thrust acting on the dam.

(OR)

9(b)	Describe the assumptions of Moment distribution method.

10(a)	A fixed beam of span 4.5m carries a point load of 50kN at the centre. Calculate net positive bending moment.

# (OR)

10(b)  Determine the forces in all the members of the truss shown in figure by method of

B

600	600

A

6m

C

joints.

10kN

11(a)	A two span continuous beam ABC of spans 4m and 6m is fixed at A and C. Calculate the distribution factors at the joint B.

# (OR)

11(b)  Explain “Distribution factors” and “Carry over factors”.

12(a) Distinguish between a deficient frame and redundant frame with the help of an example.

# (OR)

12(b) Briefly explain the method of sections of determining the forces in the members of a frame.

# PART-C

Answer FOUR questions. Each question carries five marks.	4 x 5 = 20

Marks

13(a) A trapezoidal masonry dam 6m high, 1.5m wide at its top and 3.5m wide at its bottom retains water on its vertical face. What are the maximum and minimum stresses at the base when the reservoir is full? Take ωm= 22kN/m3 and ωw= 9.81kN/m3.

(OR)

13(b)  Five members of same material are rigidly connected at O as shown in Fig.

If a moment of 120 KN-m is applied at the joint O. Find out the moments distributed among the members as per table below.E is constant.

| Member | Length in meters | Moment of inertia in mm4 |
| --- | --- | --- |
| OA | 2.0 | 4 15X10 |
| OB | 2.6 | 4 13X10 |
| OC | 2.8 | 4 18X10 |
| OD | 2.4 | 4 22X10 |
| OE | 2.0 | 4 21X10 |

14(a) A two span continuous beam ABC is simply supported at A, B and C. Span AB = 5m and span BC = 5.5m. A central point load of 40kN acts in span AB and a UDL of 10kN/m acts over span BC. Calculate fixed end moments using Moment Distribution method and draw bending moment diagram.

(OR)

14(b)

15(a) A two span continuous beam ABC is fixed at A and C. Span AB = 5m and span BC = 6m. A central point load of 20kN acts in span AB and a central point load of 40kN acts over span BC. Calculate fixed end moments using Moment Distribution method and draw bending moment diagram.

# (OR)

15(b) A two span continuous beam ABC is simply supported at A, B and C. Span AB = 6m and span BC = 4.5m. A central point load of 20kN acts in span AB and a UDL of 8kN/m acts over span BC. Calculate fixed end moments using Moment Distribution method and draw bending moment diagram.

16(a) Determine the forces in the members BC, DC and BD of the truss shown in figure by method of joints.

# (OR)

16(b) Determine by method of sections the magnitude and nature of forces induced in the members BD, CA and CE of the girder shown in the figure. Span AC=CE=EG=3m,