Department of Technical Education

State Board of Technical Education &Training (TS)

| Course Title: | Civil Engineering Drawing-II |  | Course Code : | CE- 407 |
| --- | --- | --- | --- | --- |
| Semester: |  | IV Semester | Course Group : | Practical |
| Teaching Scheme in Periods(L:T:P): |  | 15:0:30 | Credits : | 2 |
| Methodology : |  | Lecture+ Practical | Total Contact Periods : | 45 |
| CIE : |  | 60 Marks | SEE : | 40 Marks |

Pre requisites

Basic knowledge of Engineering Drawing, ability to visualise 2D and 3D views

Course Outcomes

On completion of the course the student will be able to

| CO1 | Draw and illustrate the plan, elevation and section of culverts drawings |
| --- | --- |
| CO2 | Prepare the plan, elevation and section of Bridge drawings |
| CO3 | Sketch and draw the plan, elevation and section of Earthen bunds and Tank surplus weirs drawings |
| CO4 | Develop the plan, elevation and section of Tank sluice and Canal drop drawings |
| CO5 | Illustrate and draw the plan, elevation and section of Sanitary block and Septic tank drawings |
| CO6 | Draw the plan, elevation and section of R.C.C Overhead tank and Rain water harvesting pit drawings |

Course Content and Blue Print of Marks for SEE

| Unit No | Unit Name | Hours/Periods | Questions to be set for SEE |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | R | U | A |
| 1 | Culverts and Bridges | 15 | 1 | 1 | 1 |
| 2. | Irrigation Engineering drawings | 15 |  | 2 |  |
| 3. | Public health Engineering drawings | 15 |  |  | 1 |
| Total |  | 45 | 1 | 3 | 2 |

Course Contents

UNIT 1: Culverts and Bridges			Duration: 15 Periods(L:5 – P:10)

            - Pipe culvert (Single Pipe)

            - R.C.C slab culvert with square return walls

- Two-Span R.C.C T-beam Bridge with square return walls.

- Two-Span R.C.C Bridge with splayed wing walls and Return walls.

UNIT 2: Irrigation engineering drawings		Duration: 15 Periods(L:5 – P:10)

- Earthen bunds – Two types.

          - Homogeneous type 	(ii) Non Homogeneous  type.

- Tank surplus weir with splayed wing walls.

- Tank sluice with tower head.

- Canal drop (notch type)

- Canal regulator

UNIT 3: Public health engineering drawings		Duration:15Periods(L:5 – P:10)

  - Plan and cross section of a sanitary block showing internal water supply and sanitary fittings

  - Septic tank and soak pit with details of connections

  - R.C.C overhead square tank. (four columns with all accessories).

  - Draw plan and cross section of a rain water harvesting pit for a residential building.

Recommended Books

- Civil Engineering Drawing-II   byN.Srinivasulu.

- Civil Engineering Drawing-II   by Chakravarthy

Suggested E-learning references

            - http://nptel.ac.in

            - https://www.youtube.com/watch?v=1JdAPaDHueM

Suggested Learning Outcomes

After completion of the subject, the student will be able to

CO1 - Draw and illustrate the plan, elevation and section of culverts drawings

CO2 - Prepare the plan, elevation and section of Bridge drawings

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation of Pipe culvert and identify the component   parts from the given set of specifications

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation R.C.C –slab culvert    and identify the component   parts from the given set of specifications.

  - Draw the plan,sectional elevation and cross section of Two span R.C.C   T-beam bridge with square return walls from the set of given specifications.

  - Draw the sectional elevation, plan and cross section of Two span R.C.C  bridge withsplayed wing walls and  return walls from the set of given specifications.

CO3 - Sketch and draw the plan, elevation and section of Earthen bunds and Tank surplus weirs drawings

CO4 - Develop the plan, elevation and section of Tank sluice and Canal drop drawings

  - Draw the cross section view of Earthen bunds – Two types

(i) Homogeneous type 	(ii) Non-Homogeneous type.

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation of Tank surplus weir with splayed wing walls.

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation of Tank sluice with tower head.

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation of Canal drop (notchtype)

  - Draw the plan, cross sectional elevation and longitudinal sectional elevation of Canal regulator

CO5 - Illustrate and draw the plan, elevation and section of Sanitary block and Septic tank drawings

CO6 - Draw the plan, elevation and section of R.C.C Overhead tank and Rain water harvesting pit drawings

  - Draw plan and cross section of a sanitary block showing internal water supply and sanitary fittings

  - Draw the plan, cross-section elevation of Septic tank and soak pit with details of connections

  - Draw the plan, elevation of R.C.C overhead square tank. (four columns with accessories).

  - Draw plan and cross section of a rain water harvesting pit for a residential building.

Suggested Student Activities

- Tech fest/Srujana

- Paper/Poster presentation

- Quiz

- Group discussion

- Surprise Test

- Roof rainwater harvesting - a case study

- Visit to a nearby canal, take the field data and draw the c/s of canal.

- Identify and take the details of existing tank weir/tank sluice in the vicinity of your area and draw all the views.

- Identify and take the details of existing culvert/highway bridge/railway bridgenearby and draw all the views.

- For the given data prepare a model of any one of the following.

  - Tank sluice

  - Tank weir

  - Culvert

- Railway/Highway bridge

- Prepare a 3D model of Irrigation structure in CAD software

CO-PO Mapping Matrix

|  | Basic and Discipline Specific Knowledge | Problem Analysis | Design/Development of Solutions | Engineering Tools, Experimentation and Testing | Engineering Practices for Society, Sustainability and Environment | Project Management | Lifelong Learning | Linked PO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO 7 |  |
| CO1 | 3 | 2 |  | 2 | 2 | 1 | 2 | 1,2,4,5,7 |
| CO2 | 3 | 3 | 2 | 2 | 2 | 1 | 2 | 1,2,3,4,5,6,7 |
| CO3 | 3 | 3 | 1 | 2 | 2 | 1 | 2 | 1,2,3,4,5,6,7 |
| CO4 | 3 | 2 | 2 | 1 | 2 | 1 | 2 | 1,2,3,4,5,6,7 |
| CO5 | 3 | 3 | 1 | 2 | 2 | 1 | 2 | 1,2,3,4,5,6,7 |
| CO6 | 3 | 2 | 2 | 1 | 2 | 1 | 2 | 1,2,3,4,5,6,7 |

QUESTION PAPER PATTERN FOR MID SEMESTER EXAMS

| Sl.No | Description | Level | No of Questions | Marks for each question | Choice | Total Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Part-A | Remembering(R) and Understanding(U) | 2 | 4 | Nil | 8 Marks |
| 02 | Part-B | Application(A) | 2 | 12 | 1 | 12 Marks |
| Total Marks |  |  |  |  |  | 20 Marks |

QUESTION PAPER PATTERN FOR  SEMESTER END EXAM

| Sl.No | Description | Level | No of Questions | Marks for each question | Choice | Total Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Part-A | Remembering(R) and Understanding(U) | 4 | 4 | Nil | 16 Marks |
| 02 | Part-B | Application(A) | 2 | 24 | 1 | 24 Marks |
| Total Marks |  |  |  |  |  | 40 Marks |

State Board of Technical Education and Training,Telangana

Model Question paper

DCE IV semester

Mid Semester-I Examination

Course Code: CE- 407						Duration:1 hour

Course Name: CIVIL ENGINEERING DRAWING-II		Max.Marks:20 Marks

PART-A

Answer all questions.Each question carries fourmarks			2x 4 = 8 Marks

1. Draw the cross section of pipe culvert for following particulars.

Internal dia of pipe 	      = 1000mm

External dia of pipe   = 1200mm

Width of bedding       = 1600mm

Thickness of bedding  = 300mm

Thickness of benching = 250mm

2. Draw the cross section of an abutment of slab culvert for the following data:

Bottom level of CC bed 		= +45.20m

Top level of CC bed			= +45.80m

Width of CC bed				= 2.2m

Bed level					= +47.00m

Masonry footings of abutments = Consists of two below B.L.

Footings of equal depth and offsets on rear side only

Width of first footing	= 1.8m,

Width of second footing = 1.6m

Top level of abutment = + 48.50m,

Thickness of bed block flushing

With of top abutment = 0.3 m

Top width of abutment = 0.9 m having batter on rear face

Thickness of slab = 200 mm,

Thickness of wearing course = 60mm

PART-B

Answer any one question.Each question carries twelve marks		1x12=12 Marks

3.	Draw the plan and the longitudinal section of a pipe culvert to scale 1:100 to the given Particulars:

- Drain particulars

Bed level			 : +50.350

Bed width near the pipe culvert: 1200mm

Side slopes of drain		: 1:1

General G.L. near the drain	: +51.550

Bed pitching and side slope revetment on both U/S and D/S = 200mm, rough

Stone bed pitching to a length of 1200 mm shall be provided both on U/S and D/S.

A toe of same width (200mm) shall be taken to a level of +50.00 at the end of bed pitching.

Sides slope revetment shall be with 200mm size rough stone along the slopes to a length of 1200mm both on US/ and D/S from both B.L to general G.L.

- Pipes Details:

Internal diameter of C.C. pipe 		: 1000mm

External diameter of C.C. pipe 	: 1200mm

Bedding for the Pipe  			: 250mm CC

Benching for the pipe 			: 300mm CC

Width of both bedding and pitching 	: 1800 mm

Bottom level of C.C. bedding 		: +50.00m

No. of pipes				: One

- Head walls:

At the end of pipe, two head walls are provided with brick masonry with the

Following details:

Length of head wall			: 7200mm

Bottom level of head wall		: +49.10

Top level C.C. bed provided under head walls	: +49.10

Bottom level of C.C. bed provided under head walls: +48.80

Width of C.C. Bed				      : 1800mm

Bottom width of head wall			     : 1200mm

Profile of head wall = outer surface vertical and earth fill face having a batter so that the top width = 450mm.

Top level of head wall		: +52.00

- Earth fill and Embankment:

Formation width 		: 10,000 mm

Side slopes 		: 2 horizontal to 1 vertical

Formation level 		: +54.00

Height of earth fills	: top level of formation – top level of pipe = 54.00 - 51.450 =

2.550m

- Guide stones on both the sides of formation:

450mm x 450mm square guide stones are provided at distance of 450mm from extreme   edges of formation. These stones are taken t o a depth of 600mm below formation level and extended to a height of 700mm above formation level at 3000mm C.C.

4.Draw the following view of a small T-beam and deck slab bridge of two spans across a canal to a scale of 1: 50 from the given specifications. The type of structure is box type

i) Half cross-section & half elevation along the road.

Specifications:

i). 	General

No. of spans			=2

Clear width of each span	            =3.5m

Bed level of canal		           =+50.00

F.S.L				=+52.00

Free board			            =1.0m

Road formation level		=+53.58

Side slopes of canal		=1:1

Road width between parapets	=4.40m

Road width between kerbs	=4.10m

Kerb one either side 		=150mmx150mm

Wing walls			=Return type

ii).	Foundation:

The depth of foundation is same for abutments, wing walls and pier and is taken to level of + 49.30.

Depth of C.C bed below abutments, wing walls and pier is same and equal to 400mm; i.e. top level of C.C bed =+49.70 and bottom level of C.C bed =+49.30.

Width of CC bed:

a. For abutments		=1700mm

b. for wing walls			=1400mm

c. For pier			=1000mm

Bottom width of stone masonry abutment at +49.70 level = 1300mm: length = 5.9m. Bottom width of stone masonry wing walls at + 49.70 level = 1000mm Length

(as measured on outer side ) = 3.9m

iii).	 Pier:

Width of stone masonry for pier is 600 mm and is same throughout its height

(up to bed block ). Cut water sharp to 60⁰ and ease water semi-circular shape are provided.

iv). 	Top width of abutments and wing walls:

The water face is vertical and rear side (earth retaining side) has a batter both forabutments and wing walls.

Top width of abutment = 700mm

Top width of wing wall =500mm

v).   T-beams:

Three no. of T-beams are provided one at the centre and one on either side at 2m c/c, having equal overhang of deck slab on both sides.

Width of rib =250 mm

Depth of rib =500 mm

Bearing for T-beams over abutments =500mm

vi).	C.C bed blocks:

T-beams are laid over C.C bed blocks provided over the abutments and wing walls.

Size of bed block is 600mmx600mmx150mm and is laid below the ribs of T-beam.

vii).	R.C.C. deck slab:

200mm thick R.C.C (1:2:4) slab is provide with 20mm thick wearing course

(C.C 1: 1 ½:3) with 8mm size stone chips. Width of R.C.C deck slab = 4.9m.

viii).	Parapet:

Brick masonry parapet 300mm square pillars with 50mm projecting copings on eitherSide at top are provided, one at middle and one each at the ends.

R.C.C posts at 1m	Clear spacing of size 100x 100 x 750mm with 25mm dia. Pipe, hand rails comprises theParapet.

ix.)	Side revetment:	Canal sides which are at 1:1 slope are provided with rough stone revetment 300mm thick over a gravel bed of 150mm thick for sufficient length. The toe wall for revetment is taken to a level of +49.40 with its bottom width equal to 450mm.

Note: Any other data required may be assumed suitably.

State Board of Technical Education and Training,Telangana

Model Question paper

DCE IV semester

Mid Semester-IIExamination

Course Code: CE- 407						        Duration:1 hour

Course Name: CIVIL ENGINEERING DRAWING-II		Max.Marks:20 Marks

PART-A

Answer all questions. Each question carries four marks			2x4 = 8 Marks

1. Sketch the central section along the tank bund of the well of atower head sluice from the following :

Internal dia = 1· m

Height of well = 4·2 m

Thickness of well staining = 0·45 m from top to a depth of2·0 m and 0·60 m for the remaining

CC foundation = 0·5 m thick with 0·3 m offset

Thickness of slab = 150 mm thick

Wooden shutter = 0·70 m wide × 1·2 m deep

Provide suitable rod and gearing rearrangement for the wooden shutter.

2. Draw the longitudinal section of a canal drop and name the component parts.

PART-B

Answer any one question							1x12=12 Marks

3. Draw the cross-section of a homogenous earthen bund with thefollowing specifications to a scale of 1 : 100.

Top width of bund = 1·5 m

TBL = +57·00

General ground level = +50·00

Stripped ground level = +49·70

Side slopes = 1½ : 1 on U/S and 2 : 1 on D/S

Key trenches = 1·2 m wide and 0·6 m deep at 4·0 m C/C

Protection of upstream face of the bund :

The upstream face of the bund is provided with 300 mmthick rough stone revetment over 15 mm thick gravel backing.

This revetment is founded on rough stone wall 1·0 m wide1·0 m deep

Protection of a downstream toe of the bund :

A rock toe with 300 m rough stone boulders is provided with900 mm top width and top level being at +51·20.

Side slopes of rock toe = 1 : 1

Sand filter = 200 mm thick on rear side and at the bottom ofthe rock toe

Toe drain = A longitudinal drain is provided with bottom width1·0 m and side slopes 1 : 1. This is in line with the outersurface of rock toe and taken to a level of +49·00

Rough stones of 300 mm thick are used for side revetment andbed pitching of toe drain

4. Draw the longitudinal section of a tank sluice with tower headto a scale of 1: 50.

(a) Tank bund :

Top width = 1·8 m

TBL = +163·500 m

MWL = +162·000

FTL = +161·300

Bed level = +159·100

Side slopes = 1½ : 1 on U/s and 2 : 1 on D/s

(b)Tower head :

Internal diameter = 1·2 m

Top of RCC slab over well = +162·50

Thickness of well staining = 450 mm from top to adepth of 2 m and 600 mm for the remaining height

Opening = 600 mm dia opening is provided in the CCdiaphragm 75 mm thick for allowing water into thebarrel.

Shutter = Wooden shutter 750 mm wide, 1500 mmdepth and 50 mm thick is provided for regulating water

Foundation for well = 3·0 m dia and 600 mm thick

(c) Sluice barrel :

Internal dimensions = 750 mm wide × 1·0 m height

Roof for barrel = RCC roof slab 150 mm thick

Side walls of the barrel = 450 mm thick at top and600 mm thick at bottom with water face vertical

CC foundation = 450 mm thick and 2550 mm wide islaid under barrel

(d) Lead chamber :

Length of wing walls = 1·8 m (horizontal distance)

Distance between wing walls inside to inside at thereceiving end = 2·0 m

Thickness of walls = 450 mm at top and 600 mm atbottom with water face vertical

Profile of wing walls = Wing walls start from +

(e) Stilling cistern :

Internal dimensions : 3·0 m × 3·0 m

Side walls = All the side walls including the outer 160·25(top of barrel slab) at the entrance of the barrel andslopes down to the bed level +159·10.

The slope fromGL to bed level is 1½ : 1.

wallhaving an opening of 600 mm for discharging waterinto field channel are 600 mm thick at bottom and450 mm thick at top, having batter on rear side. Thesewalls are taken to canal bund level +160·75

(f) Canal particulars :

Bed width = 600 mm

Side slopes = 1:1 on water side and 1½:1 on rear sideup to GL

Bed level = +159·10

Width of canal bund = 900 mm

Canal bund level = +160·750

(g) Rough stone revetment :

(i) 450 mm rough stone revetment is provided on U/s over150 mm thick gravel backing from bed level to TBL

(ii) Sides of canal are provided with 300 mm thick roughstone revetment over 150 mm thick gravel backing for alength of 1·5 m. Bed pitching is also provided in thecanal to a length of 1·20 m with 300 mm size roughstone and a toe is provided at its end and taken to adepth of 600 mm below bed level of canal.Width of toe = 300 mm

(h) General ground level at the site = +159·50

State Board of Technical Education and Training,Telangana

Model Question paper

DCE IV semesterEnd Examination

Course Code: CE- 407						        Duration:2 hours

Course Name: CIVIL ENGINEERING DRAWING-II		        Max.Marks:40

PART-A

Answer all questions.Each question carries fourmarks 4x4=16Marks

1. Sketch the section at support of an RCC slab bridge showing bed block and abutment cross-section and name the parts.

2. Draw the longitudinal section of the body wall of a canal drop with the following data :

Length of body wall = 9·0 m

Top of notch pier = + 45·00 m

Top of body wall = + 44·00 m

Top of CC foundation = + 42·80 m

Bottom of CC foundation = + 42·20 m

Offset of CC foundation = 0·3 m on either side

Notch = Trapezoidal shape with bottom width 0·6 mand side slopes = 1 : 1

3. Draw the sectional plan of RCC overhead tank from the data givenbelow :

Size of water tank = 4500 mm × 4500 mm

Thickness of sidewalls = 200 mm

Columns’ size = 400 mm × 400 mm

Size of column footings = 1500 mm × 1500 mm

Size of brace beams = 300 mm × 300 mm

4. Draw the cross section of a water harvesting pit

PART-B

Answer any one question. Each question carries twenty four marks.	1x24=24 Marks

5. Draw the longitudinal section of a tank sluice with tower headto a scale of 1: 50.

(a) Tank bund :

Top width = 1·8 m

TBL = +163·500 m

MWL = +162·000

FTL = +161·300

Bed level = +159·100

Side slopes = 1½ : 1 on U/s and 2 : 1 on D/s

(b) Tower head :

Internal diameter = 1·2 m

Top of RCC slab over well = +162·50

Thickness of well staining = 450 mm from top to adepth of 2 m and 600 mm for the remaining height

Opening = 600 mm dia opening is provided in the CCdiaphragm 75 mm thick for allowing water into thebarrel.

Shutter = Wooden shutter 750 mm wide, 1500 mmdepth and 50 mm thick is provided for regulating water

Foundation for well = 3·0 m dia and 600 mm thick

(c) Sluice barrel :

Internal dimensions = 750 mm wide × 1·0 m height

Roof for barrel = RCC roof slab 150 mm thick

Side walls of the barrel = 450 mm thick at top and600 mm thick at bottom with water face vertical

CC foundation = 450 mm thick and 2550 mm wide islaid under barrel

(d) Lead chamber :

Length of wing walls = 1·8 m (horizontal distance)

Distance between wing walls inside to inside at thereceiving end = 2·0 m

Thickness of walls = 450 mm at top and 600 mm atbottom with water face vertical

Profile of wing walls = Wing walls start from +160·25

(top of barrel slab) at the entrance of the barrel andslopes down to the bed level +159·10. The slope fromGL to bed level is 1½ : 1.

(e) Stilling cistern :

Internal dimensions : 3·0 m × 3·0 m

Side walls = All the side walls including the outer wallhaving an opening of 600 mm for discharging waterinto field channel are 600 mm thick at bottom and450 mm thick at top, having batter on rear side. Thesewalls are taken to canal bund level +160·75

(f) Canal particulars :

Bed width = 600 mm

Side slopes = 1:1 on water side and 1½:1 on rear sideup to GL

Bed level = +159·10

Width of canal bund = 900 mm

Canal bund level = +160·750

(g) Rough stone revetment :

(i) 450 mm rough stone revetment is provided on U/s over150 mm thick gravel backing from bed level to TBL

(ii) Sides of canal are provided with 300 mm thick roughstone revetment over 150 mm thick gravel backing for alength of 1·5 m. Bed pitching is also provided in thecanal to a length of 1·20 m with 300 mm size roughstone and a toe is provided at its end and taken to adepth of 600 mm below bed level of canal.

Width of toe = 300 mm

(h) General ground level at the site = +159·50

6. Draw the sectional elevation and plan of a square RCCoverhead tank with the following data to a scale of 1: 50 :

Height of the tank (from GL to bottom of the tank, i.e., top

of floor slab or base slab) = 9·0 m

Size of tank = 5·0 m×5·0 m×1·75 m

Thickness of RCC side walls = 200 mm

Thickness of RCC base/floor slab = 200 mm

Thickness of RCC roof slab = 110 mm

Size of RCC column = 400 mm×400 mm

No. of RCC column = 4 no. (one at each corner)

Size of RCC brace beams = 400 mm×350 mm

Spacing of brace beams = 3·0 m c/c

Depth of RCC footing below ground level = 2·0 m

Size of footing at base = 1·6 m×1·6 m

Thickness of footing at column face = 500 mm

Thickness of footing at the end = 200 mm

Thickness of levelling course below the footing = 200 mm,(1: 4 : 8) plain concrete

Size of ring beam below base slab = 400 mm×450 mm

Dia. of inflow pipe = 100 mm

Dia. of outflow pipe = 75 mm

Size of manhole cover = 600 mm×450 mm

Show the pipe connections; ladder, water level indicator,ventilating arrangements etc. Assume any other data suitably ifneeded.