Department of Technical Education

State Board of Technical Education &Training (TS)

| Course Title: | Structural Engineering Drawing |  | Course Code : | CE-408 |
| --- | --- | --- | --- | --- |
| Semester: |  | IV Semester | Course Group : | Practical |
| Teaching Scheme in Periods(L:T:P): |  | 15:0:30 | Credits : | 2 |
| Methodology : |  | Lecture+ Practical | Total Contact Periods : | 45 |
| CIE : |  | 60 Marks | SEE : | 40 Marks |

Pre requisites

Course Outcomes

Basic knowledge of Engineering Drawing and Design principles of RCC , ability to visualize 2D and 3D.

| CO1 | Explain structural planning of building and marking of frame components |
| --- | --- |
| CO2 | Prepare detailed structural drawings of Beams and Lintels |
| CO3 | Prepare detailed structural drawings of one way and two way R.C.C Slabs |
| CO4 | Prepare detailed structural drawings of one way continuous R.C.C Slabs and T-beam |
| CO5 | Prepare detailed structural drawings of Column with square footing |
| CO6 | Prepare detailed structural drawings of stair cases |

Course Content and Blue Print of Marks for SEE

| Unit No | Unit Name | Periods | Questions to be set for SEE |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | R | U | A |
| 1 | Structural Planning And marking of Frame Components | 15 | 1 |  | 1 |
|  | R.C.C Drawings- Beams and Lintels |  |  | 1 |  |
| 2 | R.C.C Drawings – Slabs | 15 |  |  |  |
| 3 | R.C.C Drawings- Columns with footing, Stair cases and Earthquake resistant structures | 15 |  | 1 | 1 |
|  | Reading and interpretation of drawings |  |  | 1 |  |
| Total |  | 45 | 1 | 3 | 2 |

Course Contents

UNIT 1: Structural planning of a building and marking of Frame components and R.C.C  Drawings- Beams and Lintels

Duration: 15 Periods.(L:5-P:10)

  - Draws the position of columns, beams, slabs, stairs and footing in a given line diagram of building-guidelines and space standards for barrier free built environment

  - Prepare member reference scheme of given building following Column reference scheme & Grid reference scheme as per IS: 5525 – (recommendations for detailing of reinforced concrete works). &  SP:34

  - Singly reinforced simply supported rectangular beam

  - Lintel cum sunshade

UNIT 2:R.C.C  Drawings- Slabs			Duration: 15 Periods.(L:5-P:10)

  - Simply supported one-way slab.

  - Two-way slab simply supported corners not held down.

  - Two-way slab simply supported corners held down.

  - One-way continuous slab and T-beam (with details of slab and T-beam)

UNIT 3:R.C.C Drawings -Columns with footing, Stair cases and Earthquake resistant                 structures and Read and interpret the drawings

Duration: 15 Periods.(L:5-P:10)

- Column with square footing of uniform thickness.

- Stair case – stairs spanning longitudinally (Dog legged stair case)

- Frame showing the details of reinforcement for earth quake resistant structures

- Take the details of reinforcement from the given drawings

- Preparation of Schedule of reinforcement for a given structural drawing.

Recommended Books

- Hand book on Concrete reinforcement and detailing (IS CODE – SP 34)	IS 5525: Recommendations for detailing of reinforcement in reinforced concrete worksby Bureau of Indian Standards

Suggested E-learning references

1. http://nptel.ac.in

Suggested Learning Outcomes

Upon completion of the subject the student shall be able to

CO1 - Explain structural planning of building and marking of frame components

CO2 - Prepare detailed structural drawings of Beams and Lintels

  - Understand Positioning and Orientation of beams and column base upon the guidelines and space standards for barrier free built environment.

  - Understand Spanning of slabs , layout of stairs

  - List types of footings

  - Prepare member reference scheme of given building following

      - Column reference scheme and

      - Grid reference scheme (Scheme recommended by IS:5525 –recommended for detailing of reinforced concrete works and SP-34)

  - Draw the longitudinal section and cross sections of singly reinforced simply supported rectangular beam.

  - Prepare schedule of reinforcement and quantity of steel for singly reinforced    simply supported rectangular beam

  - Draw the longitudinal and cross section of lintel cum sunshade

  - Prepare schedule of reinforcement and quantity of steel for lintel cum sunshade

CO3 - Prepare detailed structural drawings of one way and two way R.C.C Slabs

CO4 - Prepare detailed structural drawings of one way continuous R.C.C Slabs and T-beam

  - Draw the plan and longitudinal section of one-way slab showing reinforcement details.

  - Prepare schedule of reinforcement and quantity of steel for one-way slab showing

reinforcement details

  - Draw the details of reinforcement of two-way simply supported slab with corners not held down condition.

  - Draw top and bottom plan and section along short and long spans of two way simply supported slab with corners not held down condition

  - Draw the details of reinforcement of two-way simply supported slab with corners held down conditions.

  - Draw top and bottom plan and section along short and long spans have to be drawn (Scheduling of reinforcement is not necessary).

  - Draw the details of reinforcement of one-way continuous slab along with T- beam with details of slab and T-beam (plan and section of continuous slab and longitudinal section of T-beam have to be drawn). (Scheduling of steel is not necessary)

CO5 - Prepare detailed structural drawings of Column with square footing

CO6 - Prepare detailed structural drawings of stair cases

  - Draw the details of column and square footing (plan and sectional elevation) prepare schedule of reinforcement of column and footing and quantity of steel required.

  - Draw the reinforcement details of dog legged stair case (section only) prepare schedule of reinforcement for one flight including landing.

  - Understand the details of reinforcement from the given drawings

  - Fill in the details of reinforcement in a drawing.

Suggested Student Activities

- Tech fest/Srujana

- Paper/Poster presentation

- Quiz

- Group discussion

- Surprise Test

CO-PO Mapping Matrix

|  | Basic and Discipline Specific Knowledge | Problem Analysis | Design/Development of Solutions | Engineering Tools, Experimentation and Testing | Engineering Practices for Society, Sustainability and Environment | Project Management | Lifelong Learning | Linked PO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO 7 |  |
| CO1 | 2 | 2 | 2 |  | 1 |  | 2 | 1,2,3,5,7 |
| CO2 | 2 | 2 |  |  | 1 |  | 2 | 1,2,5,7 |
| CO3 | 2 | 2 |  |  | 1 |  | 2 | 1,2,5,7 |
| CO4 | 2 | 2 | 1 |  | 1 |  | 2 | 1,2,3,5,7 |
| CO5 | 2 | 2 |  |  | 1 |  | 2 | 1,2,5,7 |
| CO6 | 2 | 2 | 1 |  | 1 |  | 2 | 1,2,3,5,7 |

QUESTION PAPER PATTERN FOR MID SEMESTER EXAMS

| Sl.No | Description | Level | No of Questions | Marks for each question | Choice | Total Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Part-A | Remembering(R) and Understanding(U) | 2 | 4 | Nil | 8 Marks |
| 02 | Part-B | Application(A) | 2 | 12 | 1 | 12 Marks |
| Total Marks |  |  |  |  |  | 20 Marks |

QUESTION PAPER PATTERN FOR  SEMESTER  END EXAM

| Sl.No | Description | Level | No of Questions | Marks for each question | Choice | Total Marks |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Part-A | Remembering(R) and Understanding(U) | 4 | 4 | Nil | 16 Marks |
| 02 | Part-B | Application(A) | 2 | 24 | 1 | 24 Marks |
| Total Marks |  |  |  |  |  | 40 Marks |

State Board of Technical Education and Training, Telangana

Model Question paper

DCE IV Semester

Mid Semester-I Examination

Course Code: CE-408						      Duration: 1 Hour

Course Name: Structural Engineering Drawing			      Max.Marks: 20 Marks

PART-A

Answer all questions. Each question carries four marks                                              2x4=8Marks

Instructions:  (1) To be drawn not to scale.

(2) Assume suitable data, if necessary.

1. State any two guiding principles for positioning in a structural planning of a building for the following.   a) Columns b) Beams.

2. Mark the position of columns in the given diagram and name them as per ‘Grid Reference

Scheme’

| ROOM 4000 X 3600 MM | ROOM 2000 X 3600 MM |
| --- | --- |

PART-B

Answer any one questions.Each question carries twelve marks		1 x 12=12 Marks

Instructions :

(1) Draw all questions to scale.

(2) Any missing data may be assumed suitably

3. A singly reinforced rectangular beam of width 230mm and gross depth 400mm is simply supported over a clear span of 3000 mm. Bearing on each side is 200 mm.It is reinforced with 4 nos 16mm dia  bars with a clear cover of 40 mm and 2 anchor bars of 12 mm dia are provided.

Middle bars of tension reinforcement are cranked through 450at a distance of 0.1 times the clear span from the face of the support.To resist shear two legged stirrups of  8 mm dia at 225 mm c/c are provided. The end covers are 40mm.The materials used were M20 grade concrete and deformed bars of grade Fe415.

Draw the longitudinal section for the above specifications to a scale of 1:15

4. A RCC lintel with sunshade has the following specifications :

Lintel

Clear span of Lintel—1·50 m

Width of wall—230 mm

Size of Lintel—230 mm × 200 mm

Bearing on walls—150 mm

Reinforcement of Lintel

Main reinforcement—4 Nos. of 12 mm dia (middle two bars cranked at 45° at 220 mm fromface of the support)

Hanger bars—2 Nos. of 10 mm dia

Stirrups—6 mm dia 2 legged at 180 mm c/c throughout

Sun shade:

Projection of sunshade from face of the wall—600 mm

Thickness of sunshade—80 mm at fixed end. 60 mm at free end

Reinforcement of sunshade:

Main bars—10 mm dia bars at 150 mm c/c

Distribution steel—8 mm dia @ 180 mm c/c

Covers:

Bottom clear cover in lintel:30 mm

Top clear cover in sunshade:20 mm

All the remaining covers :25 mm

Draw the cross section of lintel with sunshade with a suitable scale

State Board of Technical Education and Training, Telangana

Model Question paper

DCE IV Semester

Mid Semester-IIExamination

Course Code: CE-408						      Duration: 1 Hour

Course Name: Structural Engineering Drawing			      Max.Marks: 20 Marks

Answer all questions.

PART-B

Answer any one question. Each question carries twelve marks.	1 x 12 = 12 Marks

Instructions:

(1) Draw all questions to scale.

(2) Any missing data may be assumed suitably

- Draw to scale of 1:20 the bottom plan of reinforcement of a simply supported Two  way  slab with the following specifications:

Size of the room    : 4.2 m x 5.0 m

Edge conditions      : simply supported, corners not held  down

Overall depth of the slab:140mm

Bearing on walls     :230mm

Reinforcement:

Steel for shorter span = #12 at 220mm c/c (alternate bars are cranked at a distance of 400mm from face of support)

Steel for longer span = # 10 at 250mm c/c (alternate bars are cranked at a distance of 500mm from the face of the support)

Covers:

Bottom clear cover = 20mm

Top clear cover       =  20mm

Side covers               = 25mm

Materials:

Concrete : M20 grade

Steel : Fe415

- Draw the reinforcement details of a simply supported two way slab whose corners are held down with the following specifications.

Specifications:

Size of the room  = 4.8 m x6.2 m

Edge conditions   = simply supported, corners held down

Overall depth of the slab  =160mm

Bearing on walls   = 300mm

Reinforcement Along shorter span:

In middle strip = 12m dia. at 180mm c/c

In edge strip    = 12mm dia. at 300mm c/c

(Alternate bars are cranked at a distance of 480 mm from the face of  the support)

ReinforcementAlong  longer span

In middle strip=12mm dia at 220 mm C/C

In edge strip=12mm dia at 300 mm C/C

(Alternate bars are cranked at a distance of 620mm  from the face of the support)

Torsion reinforcement

In the form of mesh 990 mm x990 mm in four layers with 8mm dia. bars 10 nos in each layer at all four corners

Covers:

Bottom clear cover = 20mm

Top clear cover  =20mm

Side covers =25mm

Materials;

Concrete: M20 grade concrete

Steel : Fe415 steel

Draw to a scale of 1:25 bottom plan of the reinforcement

State Board of Technical Education and Training, Telangana

Model Question paper

DCE IV Semester

Semester EndExamination

Course Code: CE-408						      Duration: 2 Hour

Course Name: Structural Engineering Drawing			 Max.Marks: 40 Marks

PART-A

Answer all questions. Each question carries four marks                                           4x4=16Marks

Instructions:  (1) To be drawn not to scale.

(2) Assume suitable data, if necessary

- State any five guiding principles for positioning of columns in a structural planning of a building.

- A singly reinforced rectangular beam of width 230mm and gross depth 400mm is simply supported over a clear span of 3000 mm. Bearing on each side is 200 mm.It is reinforced with 4 nos 16mm dia  bars with a clear cover of 40 mm and 2 anchor bars of 12 mm dia are provided.

Middle bars of tension reinforcement are cranked through 450at a distance of 0.1 times the clear span from the face of the support.To resist shear two legged stirrups of  8 mm dia at 225 mm  c/c are provided. The end covers are 40mm.The materials used were M20 grade concrete and deformed bars of grade Fe415.

Draw the cross section of the beam at middle span for the above specifications

- Draw the longitudinal section of an isolated square footing for a column with the following specifications.

Size of the column = 400x400mm

Size of the footing=2100x2100mm

Thickness of the footing=450mm

Base course thickness: 150mm with P.C.C, 1:2:4

Reinforcement for footing :

12mm dia at 160 mm c/c in both the directions at bottom with a clear cover of 50mm. Thehorizontal lap length of the column reinforcing bar is 500mm each.

Reinforcement for the column:

Main bars: 16mm dia bars 4nos

Lateral ties:  8mm dia ties at 220mm c/c ,

All covers 50mm

Materials Used:

M20 Grade Concrete and Fe415 Steel

4. Prepare the bar bending schedule and find the quantity of steel required for the mainreinforcement for lintel shown in the figure below. Top and bottom covers are 25mm and all the remaining covers are 40mm

PART-B

Answer any one question. Each question carries Twenty four marks

1 x 24= 24 Marks

Instructions :

(1) Draw all questions to scale.

(2) Any missing data may be assumed suitably

- Draw the reinforcement details of a simply supported singly reinforced RCC beam with the following specifications:

Specifcations;

Clear span of the beam :3800mm

Bearing on either side :200mm

Width of the beam :300mm

Overall depth of the beam :500mm

Materials:

Concrete :M20 grade

Steel : Fe 415 steel

Reinforcement :

Bars in tension : 4 # 16,out of which 2 middle bars  are cranked at a distance of 400mm from the face of the support at 45 °

Hanger bars :2#12

Stirrups: #8,two legged stirrups at 200mm c/c throughout .

Covers:

Top and bottom clear cover :25mm

Side clear cover:40mm

Draw the following views to a scale of 1:20

- Longitudinal section of beam

- Cross section at the mid span

6.	Draw the reinforcement details of a longitudinally spanned doglegged stair case with the following specifications to a scale of 1:20.

Specifications:

Size of the stair case room: 2500x4000mm

Height of the floor : 3600mm

Tread(T) : 270mm

Rise(R) : 150mm

Thickness of the waist slab : 200mm

Bearing in the wall: 200mm

Thickness of the wall: 300mm

Projection into the basement :300x300mm

Width of the staircase:1200mm

Reinforcement:

Main bars:12mm dia bars at 160mm c/c (Alternate bars are cranked at L/7 from the bottom end)

Distribution bars: 8mm dia bars at170mm c/c

Additional bars of 12mm dia at 140mm c/c at the junction of landing slab with the waist slab. Project this bars through a distance of 1000mm from the junction point downwards parallel to the waist slab.

Covers:

Bottom clear cover= 20mm

All the remaining covers= 25mm

Materials used:

Concrete: M20 grade

Steel: Fe 415