ME-108 – MANUFACTURING ENGINEERING LAB-I

| Course Title | Manufacturing Engineering Lab-I | Course Code | ME-108 |
| --- | --- | --- | --- |
| Semester | I Year | Course Group | Practical |
| Teaching Scheme in Periods (L: T: P) | 30:00:60 | Credits | 2 |
| Methodology | Lecture+ Practice | Total Contact Periods : | 90 Periods |
| CIE | 60 Marks | SEE | 40 Marks |

Pre requisites

This course requires the basic skills of handling workshop tools; this course also requires the basic knowledge of applied science and mathematics at secondary school level.

| Unit No | Unit name | Periods | Questions for SEE |  |  | Marks Weightage | % Weightage |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Handling | Manipulation | Precision |  |  |
| 1 | Fitting shop | 15 | 10 | 15 | 15 | 40 | 100 |
| 2 | Forging shop | 15 | 10 | 15 | 15 | 40 | 100 |
| 3 | Carpentry shop | 15 | 10 | 15 | 15 | 40 | 100 |
| 4 | Sheet metal shop | 15 | 10 | 15 | 15 | 40 | 100 |
|  | Total | 60 |  |  |  |  |  |
| Note: - Student can answer any one question out of 4 questions. - To pass in practical Exam student should acquire 50% marks in both CIE and SEE separately and CIE & SEE put together. - If the students acquire less than 50% in CIE, accordingly the students have to acquire more than 50% in SEE to get overall 50 % to pass |  |  |  |  |  |  |  |

On completion of course the student should be able to;

| CO1 | Identify and use the tools and equipment in fitting Shop |
| --- | --- |
| CO2 | Acquire skill in basic fitting operations |
| CO3 | Identify and use the tools to perform forging operations |
| CO4 | Identify and use the tools to perform carpentry operations |
| CO5 | Identify and use the tools to perform operations in sheet metal shop |

COURSE CONTENTS(MID – 1 & 2)

#### FITTING SHOP

- Introduction to fitting shop and identification of tools

- Cutting of M.S. Flats of 6 mm thick with hack saw

- Marking and chipping on Mild – steel flat 12 mm thick.

- Marking, cutting, drilling, Chamfering and tapping on a M.S. Flat 12 mm thick.

- Assembling of two pieces, Matching by filing (L/V Shape)

FORGING SHOP

- Introduction to forging shop and identification of tools

- Conversion of round to square.

- Conversion of round to Hexagon.

- Preparation of chisel from round rod.

- Preparation of ring and hook from M.S. round.

#### CARPENTRY SHOP

- Introduction to carpentry shop and identification of tools

- Cutting, Planning of wood with hand saw.

- Chiseling of wood and Lap joint

- T- Joint

- Preparation of Dovetail joint.

#### SHEET METAL SHOP

- Introduction to sheet metal and identification of tools

- Practice on cutting of sheet

- Formation of joints like grooved joints, locked groove joint

- Preparation of a rectangular open type tray and hallow cylinder

- Preparation of funnel

COURSE CONTENTS SEE

Turning section – List of Experiments

- Identification of parts of a lathe, safety practices and operational controls like ON/OFF, feed controls, and setting of spindle RPM

- Demo and practice on holding the work piece in three-jaw and four-jaw chucks

- To demonstrate and practice fixing an HSS single-point cutting tool in the tool post by adjusting its height to coincide with the lathe axis.

- Facing operation: To reduce the length of the workpiece by 5–10 mm using successive cuts of 1 mm depth of cut for each pass.

- Plain (straight) turning: To reduce the diameter of the workpiece by 5 mm using successive cuts of 0.5 mm depth of cut for each pass

- Calculation of taper angle for compound rest swivelling method.

- Practice of step turning, taper turning, and grooving on a lathe machine.

Welding Practice – List of Experiments (Beginner Level)

- Study of welding shop safety rules, Personal protective equipment (welding helmet, gloves, apron etc.,), fire safety, and first-aid practices

- Identification of welding equipment, tools, accessories, and consumables

- Identification and selection of welding electrodes.

- Arc striking practice on MS plate

- Straight bead laying on MS plate

- Weaving bead practice on MS plate

- Lap joint (fillet weld) in flat position

Foundry shop

- Study and identification of foundry tools and appliances
(hand tools, moulding tools, flasks, patterns, ladles).

- Preparation of moulding sand and testing of its properties
(moisture content, green strength – demonstration level).

- Green sand mould preparation using a single-piece pattern.

- Demonstrate the procedure for cutting a gating system in a mould and explain the importance of each component, pouring basin, sprue, runner, gate, and riser.

SUGGESTED LEARNING OUTCOMES

FITTING SHOP –

After completing the fitting shop experiments, the student will be able to:

- Identify and select appropriate fitting tools and equipment used in bench work.

- Cut mild steel flats to required dimensions using a hand hacksaw safely and accurately.

- Perform marking and chipping operations on mild steel flats as per given layout.

- Carry out marking, cutting, drilling, chamfering, and tapping operations on mild steel flats following standard practices.

- Assemble and match two components by filing to produce L-shape and V-shape fits with proper accuracy.

FORGING SHOP

After completing the forging shop experiments, the student will be able to:

- Identify forging tools and equipment and explain their functions.

- Convert a round bar into a square section using basic forging operations.

- Forge a round bar into a hexagonal section maintaining dimensional accuracy.

- Prepare a chisel from a round rod using appropriate forging techniques.

- Fabricate simple forged components such as rings and hooks from mild steel round bars.

CARPENTRY SHOP

After completing the carpentry shop experiments, the student will be able to:

- Identify carpentry tools and machines used for wood working operations.

- Perform cutting and planing operations on wood using hand saws and planes.

- Prepare lap joints using chiseling and fitting techniques.

- Construct T-joints with proper alignment and finishing.

- Prepare a dovetail joint demonstrating accuracy, strength, and craftsmanship.

SHEET METAL SECTION

After completing the sheet metal shop experiments, the student will be able to:

- Identify sheet metal tools and materials used in sheet metal operations.

- Perform cutting operations on sheet metal using hand tools safely.

- Form different sheet metal joints such as grooved joints and locked groove joints.

- Fabricate sheet metal components like a rectangular open tray and hollow cylinder.

- Prepare a funnel using layout, cutting, bending, and joining operations.

LATHE PRACTICE

- Identify the main parts of a lathe machine and operate basic controls such as ON/OFF switch, feed controls, and spindle speed selector while following standard safety practices.

- Select and mount workpieces correctly in three-jaw and four-jaw chucks ensuring proper alignment and secure clamping.

- Fix and adjust an HSS single-point cutting tool in the tool post by setting the tool height accurately at the lathe centre axis.

- Perform facing operation to reduce the length of a workpiece by 5–10 mm using successive cuts of 1 mm depth of cut while maintaining dimensional accuracy.

- Carry out plain (straight) turning operation to reduce the diameter of a workpiece by 5 mm using successive cuts of 0.5 mm depth of cut as per given specifications.

- Calculate the taper angle required for compound rest swivelling method using given taper dimensions.

- Perform step turning, taper turning, and grooving operations on a lathe machine according to the given drawing and specifications.

WELDING PRACTICE (Beginner Level)

- Follow welding shop safety rules and use appropriate personal protective equipment (PPE) while observing fire safety and first-aid practices in the welding shop.

- Identify welding equipment, tools, accessories, and consumables used in arc welding and state their basic functions.

- Select suitable welding electrodes based on the work material and basic welding requirements.

- Strike and maintain a stable arc on an MS plate using proper arc-striking technique.

- Lay uniform straight beads on an MS plate by maintaining correct arc length, electrode angle, and travel speed.

- Perform weaving bead welding on an MS plate with controlled bead width and proper heat input.

- Produce a sound lap joint (fillet weld) in flat position as per given specifications and basic weld quality requirements.

FOUNDRY SHOP

- Identify various foundry tools and appliances such as hand tools, moulding tools, flasks, patterns, and ladles, and state their functions.

- Prepare moulding sand and observe the testing of its properties such as moisture content and green strength at demonstration level.

- Prepare a green sand mould using a single-piece pattern by following standard moulding procedures.

- Demonstrate the procedure for cutting a gating system in a mould and explain the function and importance of pouring basin, sprue, runner, gate, and riser.

REFERENCE BOOKS:

1.Manufacturing Technology (VolI )  byP N Rao (McGraw Hill)

2.Mechanical Workshop & Laboratory Manual By K. C. John

Competencies and Key competencies to be achieved by the student.

FITTING SHOP – Competencies & Key Competencies

| Experiment / Activity | Competency | Key Competencies |
| --- | --- | --- |
| Introduction to fitting shop and tools | Identify and select fitting tools for bench work | Tool identification, safe handling, workplace discipline |
| Cutting MS flats with hacksaw | Cut MS flats to specified dimensions | Measurement accuracy, hacksaw handling, safety practices |
| Marking and chipping on MS flat | Perform marking and chipping as per layout | Use of marking tools, precision, surface finish |
| Marking, cutting, drilling, chamfering, tapping | Execute basic fitting operations following standard procedures | Process sequencing, drill and tap handling, quality control |
| Filing and assembling (L/V shape) | Assemble and match components by filing | Fit accuracy, dimensional control, inspection skills |

FORGING SHOP – Competencies & Key Competencies

| Experiment / Activity | Competency | Key Competencies |
| --- | --- | --- |
| Introduction to forging shop and tools | Identify forging tools and explain their functions | Tool knowledge, heat safety, workshop discipline |
| Round to square conversion | Convert round bar into square section | Hammer control, temperature judgment, dimensional accuracy |
| Round to hexagon conversion | Forge round bar into hexagonal section | Shape control, symmetry, quality of forging |
| Preparation of chisel | Manufacture a chisel using forging operations | Tool shaping, heat treatment awareness, finishing |
| Ring and hook preparation | Fabricate simple forged components | Material handling, forming skills, inspection |

CARPENTRY SHOP – Competencies & Key Competencies

| Experiment / Activity | Competency | Key Competencies |
| --- | --- | --- |
| Introduction to carpentry shop and tools | Identify carpentry tools and machines | Tool recognition, safety practices, housekeeping |
| Cutting and planing of wood | Perform cutting and planing operations | Measurement skills, surface finishing, tool control |
| Chiseling and lap joint | Prepare lap joints accurately | Joint alignment, chiseling technique, accuracy |
| T-joint preparation | Construct T-joints with proper fit | Assembly skills, finishing quality, precision |
| Dovetail joint preparation | Produce dovetail joints with strength and accuracy | Craftsmanship, attention to detail, quality workmanship |

SHEET METAL SHOP – Competencies & Key Competencies

| Experiment / Activity | Competency | Key Competencies |
| --- | --- | --- |
| Introduction to sheet metal and tools | Identify sheet metal tools and materials | Tool handling, material identification, safety |
| Cutting of sheet metal | Perform sheet metal cutting operations | Hand tool usage, accuracy, edge finishing |
| Formation of sheet metal joints | Form grooved and locked groove joints | Joint formation skills, alignment, quality control |
| Tray and hollow cylinder fabrication | Fabricate basic sheet metal components | Layout skills, bending accuracy, dimensional control |
| Funnel preparation | Prepare a funnel using sheet metal operations | Process planning, joining techniques, product inspection |

TURNING

| S. No. | Activity | Competency | Key Competency |
| --- | --- | --- | --- |
| 1 | Identification of lathe parts, safety practices, and operational controls | Identify the main parts of a lathe machine and operate basic controls while following safety rules | Workshop safety awareness, machine familiarization, operational discipline |
| 2 | Holding workpiece in three-jaw and four-jaw chucks | Select and mount the workpiece correctly in three-jaw and four-jaw chucks ensuring proper alignment | Work holding skill, accuracy in setup, manual dexterity |
| 3 | Fixing HSS single-point cutting tool in tool post | Fix and adjust the HSS cutting tool at correct centre height in the tool post | Tool setting skill, precision adjustment, attention to detail |
| 4 | Facing operation | Perform facing operation to reduce workpiece length by 5–10 mm using specified depth of cut | Machining accuracy, dimensional control, surface finishing skill |
| 5 | Plain (straight) turning operation | Carry out plain turning to reduce diameter by 5 mm using successive cuts of 0.5 mm | Process control, measurement skill, consistency in operation |
| 6 | Calculation of taper angle for compound rest swivelling | Calculate the taper angle required for compound rest swivelling method | Analytical ability, application of machining formulas, problem-solving skill |
| 7 | Step turning, taper turning, and grooving operations | Perform step turning, taper turning, and grooving as per drawing and specifications | Multi-operation machining skill, interpretation of drawings, time management |

Top of Form

Bottom of Form

WELDING

| S. No. | Activity | Competency | Key Competency |
| --- | --- | --- | --- |
| 1 | Study of welding shop safety rules, PPE, fire safety, and first-aid practices | Follow welding shop safety rules and use appropriate PPE while working in the welding shop | Safety consciousness, risk awareness, discipline at workplace |
| 2 | Identification of welding equipment, tools, accessories, and consumables | Identify welding machines, tools, accessories, and consumables used in arc welding | Equipment familiarity, technical awareness, shop readiness |
| 3 | Identification and selection of welding electrodes | Select suitable welding electrodes based on material and welding requirements | Material knowledge, decision-making, process understanding |
| 4 | Arc striking practice on MS plate | Strike and maintain a stable arc on MS plate using correct technique | Hand–eye coordination, arc control, basic welding skill |
| 5 | Straight bead laying on MS plate | Lay uniform straight beads on MS plate maintaining correct arc length and travel speed | Bead control, consistency, manual skill development |
| 6 | Weaving bead practice on MS plate | Perform weaving bead welding on MS plate with proper width and penetration | Weld pattern control, heat management, operator skill |
| 7 | Lap joint (fillet weld) in flat position | Produce a sound lap joint (fillet weld) in flat position as per given specifications | Joint preparation skill, weld quality awareness, dimensional accuracy |

Top of Form

Bottom of Form

FOUNDRY

| S. No. | Activity | Competency | Key Competency |
| --- | --- | --- | --- |
| 1 | Study and identification of foundry tools and appliances (hand tools, moulding tools, flasks, patterns, ladles) | Identify various foundry tools and appliances and state their functions | Tool identification skill, process awareness, shop familiarization |
| 2 | Preparation of moulding sand and testing of its properties (moisture content, green strength – demonstration level) | Prepare moulding sand and observe testing of its properties such as moisture content and green strength | Material handling skill, basic testing awareness, quality consciousness |
| 3 | Green sand mould preparation using a single-piece pattern | Prepare a green sand mould using a single-piece pattern following standard moulding procedure | Mould preparation skill, manual dexterity, process sequencing |
| 4 | Demonstration of cutting a gating system and explanation of its components | Identify and demonstrate cutting of gating system components and explain the function of pouring basin, sprue, runner, gate, and riser | Understanding of metal flow, casting quality awareness, analytical thinking |

| Course Outcome (CO) |  | Cognizant Level |  | Linked Program Outcomes (PO) | Teaching periods |
| --- | --- | --- | --- | --- | --- |
| CO1 | Identify and use the tools and equipment in fitting Shop | R/U/A | 1,2,3,4,7 |  | 6 |
| CO2 | Acquire skill in basic fitting operations | R/U/A | 1,2,3,4,7 |  | 6 |
| CO3 | Identify and use the tools to perform forging operations | R/U/A | 1,2,3,4,7 |  | 11 |
| CO4 | Identify and use the tools to perform carpentry operations | R/U/A | 1,2,3,4,7 |  | 11 |
| CO5 | Identify and use the tools to perform operations in sheet metal shop | R/U/A | 1,2,3,4,7 |  | 11 |
| R: Remembering, U: Understanding, A: Applying |  |  |  |  |  |

CO-PO MATRIX

|  | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 | Mapping POs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO1 | 3 | 2 | 2 | 3 | - | - | 1 | 1, 2,3,4,7 |
| CO2 | 3 | 2 | 3 | 3 | - | - | 1 | 1, 2,3,4,7 |
| CO3 | 3 | 2 | 3 | 3 | - | - | 1 | 1, 2,3,4,7 |
| CO4 | 3 | 2 | 3 | 3 | - | - | 1 | 1,2,3,4,7 |
| CO5 | 3 | 2 | 3 | 3 | - | - | 1 | 1,2,3,4,7 |

BOARD DIPLOMA EXAMINATIONS(C-26)

Model Paper

Mid Sem-I & II

C-26, ME-108: Manufacturing Engineering Lab-I

Time: 1   Hour 							Total Marks: 20 MARKS

  - Plane the given wooden piece and cut it as per the given figure/drawing.

  - Prepare a lap joint as per the given specifications.

  - Prepare a dovetail joint with proper fit and finish.

  - Prepare a groove joint using sheet metal as per standard practice.

  - Prepare a locked groove joint using standard sheet metal techniques.

  - File the given mild steel plate and cut it according to the given drawing.

  - Convert the given round bar into a square section using forging operations.

  - Prepare a T-joint with proper alignment and finish.

  - Prepare an open tray using sheet metal as per the given dimensions.

  - Prepare a hollow cylinder using sheet metal as per the given specifications.

  - File the given mild steel plate and drill a 6 mm diameter hole as per the given drawing.

  - Convert the given round bar into a hexagonal section using forging operations.

BOARD DIPLOMA EXAMINATIONS(C-26)

Model Paper

Semester end exam

C-26, ME-108: Manufacturing Engineering Lab-I

Time: 2   Hour 							Total Marks: 40 MARKS

- Prepare a step turning job on the lathe machine as per the given drawing and specifications.

- Prepare a lathe job consisting of taper turning and grooving operations as per the given drawing.

- Prepare a lathe job consisting of taper turning and knurling operations as per the given drawing.

- Prepare a lap joint (fillet weld) on MS plates as per the given dimensions with proper edge preparation.

- Prepare a green sand mould using a single-piece pattern and provide a suitable gating system.

- Prepare a green sand mould using a split pattern and provide a suitable gating system.