ME-506- PNEUMATICS   LAB

| Course Title: | Pneumatics Lab | Course Code: | ME-506 |
| --- | --- | --- | --- |
| Semester: | V | Course group: | Core |
| Teaching Scheme(L:T:P): | 1:0:2 | Credits: | 1.5 |
| Methodology : | Lecture + Assignment | Total contact periods: | 45 |
| CIE: | 60 Marks | SEE: | 40 Marks |

Prerequisites: Basic knowledge of Metrology& pneumatics systems

COURSE OUTCOMES

On successful completion of the course, the students will be able to

| Course Outcomes |  |
| --- | --- |
| CO1 | Construct the pneumatic circuit for direct and speed control of Single & double acting cylinder |
| CO2 | Construct the pneumatic circuit for the Control of a double-acting cylinder using OR logic |
| CO3 | Construct the pneumatic circuit for the Control of a Double-acting cylinder using AND LOGIC |

LIST OF EXPERIMENTS

| S.No. | Description | No of Periods |
| --- | --- | --- |
| PNEUMATICS LAB |  |  |
| 1 | Introduction to Pneumatic Components | 3 |
| 2 | Direct control of a single-acting cylinder | 6 |
| 3 | Direct control of a double-acting cylinder | 6 |
| 4 | Speed control of a double-acting cylinder | 6 |
| 5 | Control of a double-acting cylinder with OR logic | 6 |
| 6 | Control of a double-acting cylinder with AND logic | 6 |
| Total |  |  |

REFERENCE BOOKS

- Pneumatics by SRIHARI RAO

- Pneumatic controls by FESTO

- Introduction to Pneumatic Components

  - Demonstrate the working of Pneumatic components.

  - Explain Pneumatics

  - Identify & List Symbols used in Pneumatic circuits.

  - Explain the phenomena of compressed air for transmitting power.

  - State Pascal's law.

  - List all the pneumatic applications.

  - Define Pneumatic actuator.

  - Classify the actuators.

  - Explain Linear actuators and Rotary actuators.

  - Define valve.

  - List different types of valves.

  - Explain the function of a valve.

  - Identify the components of a valve.

  - Identify ports and positions.

  - List the Applications of valves.

- Direct control of a single-acting cylinder

  - Draw the circuit diagram for actuating a single-acting cylinder.

  - Select the suitable valve.eg:3/2 valve.

  - Connect the 3/2 valve to the actuator.

  - Actuate the single-acting cylinder by operating a 3/2 valve.

- Direct control of a double-acting cylinder

  - Draw the circuit diagram for actuating a Double-acting cylinder.

  - Select the suitable valve.eg:4/2 valve.

  - Connect the 4/2 valve to the actuator.

  - Actuate the double-acting cylinder by operating a 4/2 valve.

- Speed control of a double-acting cylinder

  - Draw the pneumatic circuit for controlling the speed of a double-acting cylinder

  - Select a 4/2 or 5/2 valve and a throttle valve.

  - Connect the 4/2 valve and throttle valve as per the circuit.

  - Operate the 4/2 valve and adjust the throttle valve to control the speed of the

double-acting cylinder

- Control of a double-acting cylinder with OR logic

  - Draw the pneumatic circuit for controlling the speed of the double-acting cylinder

using   OR logic.

  - Select two 3/2 valves and a shuttle valve(OR Valve).

  - Connect 3/2 valves and OR valves to the double-acting cylinder

as per the circuit.

11.4   Operate either one of the 3/2 valves to control the speed of the double-acting cylinder.

- Control of a Double-acting cylinder with AND LOGIC

  - Draw the pneumatic circuit for controlling the speed of the double-acting cylinder

using   AND logic.

  - Select two 3/2 valves and an AND valve.

  - Connect 3/2 valves and AND valves to the double-acting cylinder as per the circuit.

  - Operate two valves simultaneously to control the speed of the double-acting cylinder.

| COURSE OUTCOMES |  | CL | Linked POs |
| --- | --- | --- | --- |
| CO1 | Construct the pneumatic circuit for direct and speed control of Single & double acting cylinder | U, A | 1,2,3,4,5,6,7 |
| CO2 | Construct the pneumatic circuit for the Control of a double-acting cylinder using OR logic | U, A | 1,2,3,4,5,6,7 |
| CO3 | Construct the pneumatic circuit for the Control of a Double-acting cylinder using AND LOGIC | R, U, A | 1,2,3,4,5,6,7 |

Legends: R = Remember; U= Understand; A= Apply and above levels (Bloom’s revised taxonomy)

CO-PO Attainment Matrix:

| COURSE OUTCOMES | PROGRAM OUTCOMES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 |
| CO1 | 1 | 3 | - | 2 | 1 | 3 | 3 |
| CO2 | 2 | 3 | - | 2 | 1 | 3 | 2 |
| CO3 | 2 | 3 | - | 2 | 1 | 3 | 2 |

Level 3- Highly Addressed, Level 2-Moderately Addressed, Level 1-Lowly Addressed.

State Board of Technical Education, Telangana State

Model Paper

ME-506 –   PNEUMATICS LAB

Mid Sem-I (CIE)

Time: 1 Hour   							Total Marks: 20 M

Note: Answer any one question.

- Connect pneumatic devices and control the direction of a Single Acting Cylinder

- Connect pneumatic devices and control the direction of a Double Acting Cylinder

- Connect the Pneumatic controls for Control of a double-acting cylinder with OR logic

- Connect the Pneumatic controls for Control of a double-acting cylinder with AND logic

- Connect pneumatic devices and control the speed of a Double Acting Cylinder

Note:	Exam should be conducted from the experiments in which students undergo training only as classes are conducted on a rotation basis.

State Board of Technical Education, Telangana State

Model Paper

ME-506 –  PNEUMATICS LAB

Mid Sem-II (CIE)

Time: 1  Hour   							Total Marks : 20 M

Note: Answer any one question.

- Connect pneumatic devices and control the direction of a single-acting cylinder

- Connect pneumatic devices and control the direction of a double-acting cylinder

- Connect the Pneumatic controls for Control of a double-acting cylinder with OR logic

- Connect the Pneumatic controls for Control of a double-acting cylinder with AND logic

- Connect pneumatic devices and control the speed of a  Double Acting Cylinder

Note:	Exam should be conducted from the experiments in which students underwent training only as classes are conducted on a rotation basis.

State Board of Technical Education, Telangana State

SEE Model Paper

ME-506 – PNEUMATICS LAB

Time: 2 Hour   							Total Marks: 40 M

Note: Answer any one question.

- Connect pneumatic devices and control the direction of a Single Acting Cylinder

- Connect pneumatic devices and control the direction of a Double Acting Cylinder

- Connect the Pneumatic controls for Control of a double-acting cylinder with OR logic

- Connect the Pneumatic controls for Control of a double-acting cylinder with AND logic

- Connect pneumatic devices and control the speed of a Double Acting Cylinder